Backend CLI

A command line utility to execute all M1-3200 Testf Fixture tests.

Backend CLI executable is “m1tfd1.cli”

The following commands are supported

lenel@m1testf4:~$ m1tfd1.cli --help

Usage: m1test [options] [command]

CLI utility to test and program M1-3200 boards

Options:

-V, --version         output the version number

-h, --help            display help for command

Commands:

m1dfu                 Start M1 in DFU mode and program bootstrap FW

tbcmd [options]       execute test board raw command

m1cmd [options]       execute M1 bootstrap raw command, make sure to run m1dfu command before

ict [options]         Executes ICT test

eeprom [options]      Program I2C EEPROM.

progmac [options]     program MAC to MP1 OTP, cannot be undone

flash [options]       program STM32M1 with the flash layout file

pingM1apps [options]  try to establish connection to port 80

cleanup [options]     pack the log and cleanup

functest [options]    executes m1-3200 function test

makelabel [options]   prints the M1-3200 Label

help [command]        display help for the command

The Command [options] are required. To find out required options execute

lenel@m1testf4:~$ m1tfd1.cli –help Command

Example:

lenel@m1testf4:~$ m1tfd1.cli ict  --help

Usage: m1test ict [options]

Executes ICT test

Options:

-s, --serial <string>          vendor serial number

-d, --debug <level>            set debug level, 0 error, 1 - info, 2 - debug

-b, --cellBatTol <cellBatTol>  tolerance for coin cell bat, valid values: new, used

-h, --help                     display help for the command

Below is an example to execute ICT test

lenel@m1testf4:~$ m1tfd1.cli ict -s 1626262 -d1 -bnew