# Why do we need to bootstrap M1-3200 for testing

M1-3200 board runs Linux operating system. The Linux operating system is programmed into the board when board passes basic voltage, gpio, and memory tests. To perform these tests a bootstrap firmware is required. The bootstrap FW is programmed into the CPU static RAM (not SDRAM). To program bootstrap firmware the board shall be placed into the DFU mode with bootstrap jumpers and then STM32 programming utility is used to program the bootstrap firmware into RAM over USB link and execute it.

The bootstrap firmware communication interface is serial. M1-3200 board has 0.1” 6 pin header for the serial terminal. Please use USB to 3.3V Serial converter to connect M1-3200 to the PC.

The M1-3200 can be controlled from the serial terminal in the bootstrap mode.

In order to get this functionality the M1-3200 board shall be connected to the PC over USB capable to do ST DFU and also Rs232  to 3.3V cable is required. Please use DTech FTDI USB to TTL Serial 3.3V Adapter Cable 6 Pin 0.1 inch Pitch Female Socket Header UART IC FT232RL Chip Windows 11 10 8 7 Linux (6ft, Black)

Since test fixture already has proper connections for the terminal and bootstrap pins, I recommend to use test fixture PC if low level debug is required.

# Programming bootstrap firmware into M1-3200 board

The easiest way to program the bootstrap firmware is to use backend CLI utility. This utility is installed on the test fixture PC. Also the test fixture has all required connections to bootstrap pins, USB and the serial port.

Execute command below

$m1tfd1.cli m1dfu

Wait until command is done. On success M1-3200 LEDs will interblink like a police car light.

Please note that bootstrap firmware will run as long as power is applied to M1-3200 board. Make sure to manually cycle the power when you are done.

# Serial Terminal

Start the serial terminal

#picocom -b 115200 /dev/ttyUSB0

To exit picocom press keys: Ctrl + A + X

Please note that Serial terminal will not “Echo” any typed character, therefore when you type there will be no characters in the terminal. This is done so the SW can use these commands in the script mode. Run the command below to enable Echo

promton

Followed by Enter. This will enable terminal Echo

Make sure to reset the board before executing Test Fixture SW because term mode is not compatible with the automated testing mode.

Now Execute command

LenelS2>help

This command will display all available commands.

ddrdatbus - Test the data bus wiring in a memory region by performing a walking 1's test at a fixed address within that region, arg: <offset from mem base hex> <memsize hex>

ddrtest - Test DDR integrity, arg: <offset from DDR base, in HEX> <size in HEX>

ddradrbus - Test the addr bus wiring by performing a walking 1's test on the bits of the address and checking for aliasing

pulsegpio - arg: <port> <pin> <width [uSec]>

confgpio - arg: <port> <pin> <mode [input or output]> <pull [up, down, none]>

getgpio - arg: <port> <pin>

setgpio - arg: <port> <pin> <value>

testrs422 - send char and receive reply

checkeeeprom - checkeeeprom arg: none

verifyeepromdata - check EEPROM data integrity

writeeepromdata - arg: <serialN> <secret> <force overwrite 1 or do not overwrite0>

testrs485 - test rs485 loopback test, needs rs485 slave to be connected in echo mode

getfwrev - return fw revision for ict test

promton - enable console promt

promtoff - disable console promt

help - display help

Most commands are self-explanatory.

To read/write GPIO you need to know GPIO port and pin values. Look at the M1-3200 schematic for.

Before GPIO can be used the GPIO needs to be configured for Input or Output.

Example: Read Ribbon Cable Pin 6 which is I2C clock pin.

- Configure the ping as output

M1-3200: confgpio a 11 output none

So, set port “a” pin “11” to be an output

- Set the value to pin

setgpio a 11 1

Below are copy of FW defined pins. Not all pins are defined by a FW. For example aioS? pins are ribbon cable pins for the blades I2C address select. Also, there are pins for the ribbon cable I@C pins. If you need other CPU pins please look into the schematics.

const ribbonCableSelectPins = [

{ name: 'aioS0', port: 'd', pin: 5, pinNameOnTestBoard: 'J5.18' },

{ name: 'aioS1', port: 'c', pin: 6, pinNameOnTestBoard: 'J5.15' },

{ name: 'aioS2', port: 'd', pin: 4, pinNameOnTestBoard: 'J5.16' },

{ name: 'aioSel7', port: 'c', pin: 10, pinNameOnTestBoard: 'J5.19' },

{ name: 'aioNrst', port: 'k', pin: 0, pinNameOnTestBoard: 'J5.17' }

];

const ribbonCableI2CPinsSlave = [

{ name: 'sda', port: 'a', pin: 12, pinNameOnTestBoard: 'J5.6' },

{ name: 'scl', port: 'a', pin: 11, pinNameOnTestBoard: 'J5.3' }

];

const ribbonCableI2CPinsMaster = [

{ name: 'sda', port: 'g', pin: 15, pinNameOnTestBoard: 'J5.6' },

{ name: 'scl', port: 'd', pin: 7, pinNameOnTestBoard: 'J5.3' }

];