# Test Vector

# Verification by testing a Golden Board

Place the Golden M1-3200 board into the test fixture and latch the test fixture cover.

Make sure the front panel Green status LED is On.

Activate SW debug mode by pressing Control M. The debug messages will be printed on the screen.

Start the ICT test by clicking ICT test Led.

[2024-03-01 00:18:43] [3023200040] info:        Commission board

[2024-03-01 00:18:44] [3023200040,     ict] error: 	M1-3200 Ethernet jack is not plugged. Check connection and retry the test.

# Step 2: Ground Pins Validation

Remove the Golden Board from the test fixture.

Open console on the Test Fixture PC. The console icon is located on the desktop.

Run the following command in the console

#picocom -b 115200 /dev/ttyACM0

For every test point in the table below below run the command

getiopin 16

The result will be

{"cmd": "getiopin", "status": true, "value": 2.603678  }

Make sure the status is true, and the value is above 2.5 (i.e. 2.5V)

| M1-3200 | Io Pin Number | Best | Name | Voltage Level | Tolerance |
| --- | --- | --- | --- | --- | --- |
| PP J5.23 | 16 | TB J J5.23 | Ribbon Ground | < 2.5V | 5% |
| PP J5.25 | 17 | TB J J5.25 | Ribbon Ground | < 2.5V | 5% |
| PP J5.20 | 18 | TB J J5.20 | Ribbon Ground | < 2.5V | 5% |
| PP J5.22 | 19 | TB J J5.22 | Ribbon Ground | < 2.5V | 5% |
| PP J5.24 | 20 | TB J J5.24 | Ribbon Ground | < 2.5V | 5% |
| PP J5.26 | 21 | TB J J5.26 | Ribbon Ground | < 2.5V | 5% |

If all test points pass the test the test is PASSED.