The Custom Test Board can be controlled over serial terminal.

Teensy USB connection to the PC is required for. The Serial terminal can be started on Test Fixture PC since Teensy 4.1 is connected to by default.

#picocom -b 115200 /dev/ttyACM0

To exit picoohm press keys: Ctrl + A + X

Please note that Serial terminal will not “Echo” any typed character, therefore when you type the will be no characters in the terminal. This is done so the SW can use these commands in the script mode. Run the command below to enable Echo

Type

term

Followed by Enter

Make sure to reset the board before executing Test Fixture SW because term mode is not compatible with the automated testing mode.

Now Execute command

LenelS2>help

This will display all available commands.

The Table 1 below lists all commands. Do not use commands in Red.

Table 1: commands

| cmd | arg | disc |
| --- | --- | --- |
| batteryloadoff | null | Disconnect battery load resistor |
| batteryloadon | null | Connect battery load resistor |
| battery off | null | Disable Battery Power In |
| batter yon | null | Enable Battery Power In |
| buzzer beep | 0 beep success, 1 beep error | Beep buzzer |
| configiopin | Pined pin config [ from 0, In, Out, IN_PUP, IN_PD, OUT_OD, In_Dis] | Do not use |
| date |  | Do not use |
| DRESG |  | Display system log buffer |
| epoch |  | Display epoch time |
| Geoffrey |  | Get FW revision |
| Ethiopia | pined | Get Io pin |
| INDEF |  | Print Io ports configuration |
| term |  | Enable local Echo |
| ll |  | Do not use |
| print |  | Print Io ports configuration |
| reboot |  | Reboot Teensy |
| semiopen | Pined state [ 0 or 1] | Set Io pin |
| set time | epoch Time | Do not use |
| targetpoweroff | null | M1-3200 power off |
| targetpoweron | null | M1-3200 power On |
| testtestpoint | [Pined] | Do not use |
| testtestpoints | [Test points group] | Do not use |
| uptime |  | Display system uptime |
| watchdog test | null | Do not use |

The Next step you will have to find pined values for the test points IDs so you can use commands: Ethiopia and semiopen.

Run the command below

LenelS2> iodef

The output is raw Jason and it is difficult to read

The Table Pinnaces below lists the output of this command.

Table 2: Io Definition

| pinnate | pined | group | Revalue (V) | pines | tintype |
| --- | --- | --- | --- | --- | --- |
| Tamper Pin | 0 | 0 | 0 | Tamper Pin | 1 |
| BatEnablePin | 1 | 0 | 1 | BatEnablePin | 1 |
| Battleaxe | 2 | 0 | 1 | Battleaxe | 1 |
| TargetPwrControl | 3 | 0 | 1 | TargetPwrControl | 1 |
| base12VAD | 4 | 3 | 12 | base12VAD | 64 |
| bat12VAD | 6 | 3 | 12 | bat12VAD | 64 |
| batChargeVAD | 7 | 3 | 13 | batChargeVAD | 64 |
| J5.5 | 8 | 1 | 6 | P6V_2 | 128 |
| J5.7 | 9 | 1 | 6 | P6V_3 | 128 |
| J5.13 | 10 | 1 | 11.8 | P12V_1 | 128 |
| J5.15 | 11 | 2 | 0 | R_S1 | 128 |
| J5.17 | 12 | 2 | 0 | First | 128 |
| J5.19 | 13 | 2 | 0 | R_S7 | 128 |
| J5.8 | 14 | 1 | 6 | P6V_1 | 128 |
| J5.14 | 15 | 1 | 11.8 | P12V_2 | 128 |
| J5.23 | 16 | 1 | 0 | R_G1 | 128 |
| J5.25 | 17 | 1 | 0 | R_G2 | 128 |
| J5.20 | 18 | 1 | 0 | R_G3 | 128 |
| J5.22 | 19 | 1 | 0 | R_G4 | 128 |
| J5.24 | 20 | 1 | 0 | R_G5 | 128 |
| J5.26 | 21 | 1 | 0 | R_G6 | 128 |
| J5.18 | 22 | 2 | 0 | R_S0 | 128 |
| J5.16 | 23 | 2 | 0 | R_S2 | 128 |
| BatCellBat | 24 | 3 | 2.8 | BatCellBat | 64 |
| TP025 | 25 | 4 | 5 | TP025 | 128 |
| TP36 | 26 | 4 | 3.3 | TP36 | 128 |
| TP31 | 27 | 4 | 3.3 | TP31 | 128 |
| TP34 | 28 | 4 | 3.3 | TP34 | 128 |
| TP35 | 29 | 4 | 3.3 | TP35 | 128 |
| TP33 | 30 | 4 | 3.3 | TP33 | 128 |
| J8.1 | 32 | 0 | 0.4 | LED_POS | 128 |
| J8.2 | 33 | 0 | 0.4 | LED_NEG | 128 |
| Buzz | 39 | 0 | 1 | Buzz | 1 |
| SwitchTargetPower | 40 | 0 | 0 | SwitchTargetPower | 2 |
| Boot Switch | 41 | 0 | 0 | Boot Switch | 2 |
| J5.3 | 44 | 2 | 3.3 | I2C_SDA | 0 |
| J5.6 | 45 | 2 | 3.3 | I2C_SCL | 0 |
| M1Boot1 | 46 | 0 | 0 | M1Boot1 | 1 |
| Sol1 | 47 | 0 | 1 | Sol1 | 1 |
| Sol2 | 48 | 0 | 1 | Sol2 | 1 |
| Lever Sensor | 49 | 0 | 1 | Lever Sensor | 2 |

Pin Type:

1 – Digital Output

2 – Digital Input

64 – Analog Input

128 – Analog Input through external analog mix

Controlling IO Pins

Two commands are available

getiopin pined

Semiopen pined value 0 or 1

Using these two commands the operator can read any Test point value or set any output pin.

Manual M1-3200 Power On and Off Control.

For M1-3200 debug is is convent to enable M1-3200 power without serial terminal commands.

The M1-3200 power is a two operation process:

- Set the boot pins (jumpers on M1-3200)

- Enable power

Two Push buttons on the Test Board can do just that.

The Blue push button (Boot Target) controls the M1-3200 boot mode, while the Orange push button (Target Power) controls the power switch.

If the Orange push button is activated while blue button is up, the M1-3200 will boot into the RFU mode, therefore it will expect that the firmware to be programmed.

When the Blue push button is pressed and the orange push button is pressed the M1-3200 will boot into the environment programmed in the M1-3200 SSD drive, i.e. Linux if the M1-3200 was already programmed.