# Test Fixture Troubleshooting guide

This document provides information on how to trouble shoot the test fixture.

This document does not provide information on how to troubleshoot or repair M1-3200 boards. For M1-3200 help contact S2 hardware team.

Reference documents:

- Test board Serial terminal.docx

- Backend CLI.docx

# Testing the test fixture

The test fixture shall be tested with the know to work “Golden” M1-3200. Please have few Golden boards for the testing.

Run Commission Test from test fixture PC front end GUI application.

If error is reported follow the information below for the troubleshooting guide.

Error which will not produce printed Error labels

- Hand help scanner issue - check USB cable, replace the scanner

- Label printer – paper jam, USB cable, power supply, replace the label printer

- Keyboard or mouse – check the battery, replace

- Ethernet Error – check cables, check Sam-E, replace SAM-E

- Test fixture power – check wiring, check power supply, replace power supply

- USB Hub – verify the Hub is working, check wiring, replace the Hub

- PC – contact LenelS2

- PC has incorrect time, or Logs sync issues – check PC internet connection

Printed Error labels

| Id | Error Code | Test | Remedy |
| --- | --- | --- | --- |
| ICT_EXCEPT | ICT-EXCEPTION | ICT | ICT test unexpected failure. Re-run the test. Contact S2 |
| TP025 | 000000000TP25 | ICT | TP25 voltage is out of tolerance. TP25 voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| TP33 | 000000000TP33 | ICT | TP33 voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| TP35 | 000000000TP35 | ICT | TP35 voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| TP34 | 000000000TP34 | ICT | TP34 voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| TP36 | 000000000TP36 | ICT | TP36 voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| TP31 | 000000000TP31 | ICT | TP31 voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J5.13 | 000000TPJ5.13 | ICT | Ribbon Cable J5.13 voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J5.8 | 0000000TPJ5.8 | ICT | Ribbon Cable J5.8 voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| BatCellBat | 000000CoinBat | ICT | Coin Cell backup battery voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J5.18 | 000000TPJ5.18 | ICT | Ribbon Cable continuity failed Check wiring, troubleshoot or replace custom test board |
| J5.15 | 000000TPJ5.15 | ICT | Ribbon Cable continuity failed Check wiring, troubleshoot or replace custom test board |
| J5.17 | 000000TPJ5.17 | ICT | Ribbon Cable continuity failed Check wiring, troubleshoot or replace custom test board |
| J5.19 | 000000TPJ5.19 | ICT | Ribbon Cable continuity failed Check wiring, troubleshoot or replace custom test board |
| J5.16 | 000000TPJ5.16 | ICT | Ribbon Cable continuity failed Check wiring, troubleshoot or replace custom test board |
| J5.5 | 0000000TPJ5.5 | ICT | Ribbon Cable voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J5.6 | 0000000TPJ5.6 | ICT | Ribbon Cable voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J5.7 | 0000000TPJ5.7 | ICT | Ribbon Cable voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J5.14 | 000000TPJ5.14 | ICT | Ribbon Cable voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J5.23 | 000000TPJ5.23 | ICT | Ribbon Cable voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J5.25 | 000000TPJ5.25 | ICT | Ribbon Cable voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J5.20 | 000000TPJ5.20 | ICT | Ribbon Cable voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J5.22 | 000000TPJ5.22 | ICT | Ribbon Cable voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J5.24 | 000000TPJ5.24 | ICT | Ribbon Cable voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J5.26 | 000000TPJ5.26 | ICT | Ribbon Cable voltage is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J8.2 | 0000000RedLed | ICT | J8.2 Red Led voltage drop is out of tolerance Check wiring, troubleshoot or replace custom test board |
| J8.1 | 0000000BlueLed | ICT | J8.1 Blue Led voltage drop is out of tolerance Check wiring, troubleshoot or replace custom test board |
| RS422 | 00000000RS422 | ICT | Ribbon Cable RS-422 echo test failed Check wiring, troubleshoot or replace custom test board |
| RS485 | 00000000RS485 | ICT | RS-485 echo test failed Check wiring, troubleshoot or replace custom test board |
| Tamper Pin | 0000000Tamper | ICT | Tamper pin test failed Check wiring, troubleshoot or replace custom test board |
| DDR3Bus | 0000DDR3-DBus | ICT | DDR3 data bus test failed Check wiring, troubleshoot or replace custom test board |
| DDR3ABus | 0000DDR3-ABus | ICT | DDR3 address bus test failed. This is not test fixture related error |
| DDR3 | 000000000DDR3 | ICT | DDR3 read/write verify test failed. This is not test fixture related error |
| EEPROM | 0000000EEPROM | ICT | EEPROM test failed |
| EEPROMUPDATE | 00UPD-0EEPROM | ICT | Update EEPROM with new data command failed. This is not test fixture related error |
| ATOP | 00BATT-SWITCH | ICT | Test Main power to battery power switching failed. Check wiring, troubleshoot or replace custom test board |
| BACH | 000BATT-CHARG | ICT | The Battery module charging voltage is out of tolerance. Check wiring, troubleshoot or replace custom test board |
| STM | 000000USB-DFU | ICT | Failed to program M1 bootstrap STM, RFU issue or BUT is down |
| APP80 | 00000APP-PING | APP | BUT Web server is down, possible APP is not running. This is not test fixture related error |
| FLASH | 0000000000FLASH | FLASH | Program FLASH failed. Check M1-3200 USB cable |
| MAC | 000000PROG-AMC | MAC | Program MAC failed. Check M1-3200 USB cable |
| MAC_CMP_ERR | 000MAC_CMP_ERR | FUN | If config reported MAC does not match OTP MAC This is not test fixture related error |
| PEN_DRIVE | 00000PEN_DRIVE | FUN | Pen drive failed. BUT USB Host port issue. Check USB cable. Replace Pen drive |
| RTC | 00000000000RTC | FUN | RTC test failed. This is not test fixture related error |
| SPI_RAM | 0000000SPI_RAM | FUN | SPI RAM test failed. This is not test fixture related error |
| FUN_EXCEPT | FUN-EXCEPTION | FUN | Functional test general failure. Contact LenelS2 |
| WDT | 0000000000WDT | FUN | Watchdog test failed. This is not test fixture related error |
| BUT_ETHER | 0000UUT_ETHER | FUN | BUT not pingable, possibly Ethernet controller or cabling issue |
| BUT_TERM | 00000UUT_TERM | FUN | BUT terminal login error, check serial link, check FW/SW revisions. Replace USB cable, Replace RS232 to USB cable |
| SSH_BRECON | 0000SSH_BRECON | FUN | Ssh connection to BUT error, check wiring or cabling issue. |
| RFU_STM | 000000DFU_STM | FUN | Program bootstrap STM file to BUT over RFU error. Check wiring, replace M1-3200 USB cable |
| I2CBus1 | 00000I2C_BUS1 | FUN | I2C Bus 1 test failed. This is not test fixture related error |
| I2CBus02 | 0000I2C_BUS02 | FUN | I2C Bus 0,2 test failed. This is not test fixture related error |
| NO_OTP_MAC | 000NO_OTP_MAC | FUN | MAC Address is not programmed in OTP. This is not test fixture HW related error |
| SERIAL_MISMATCH | SER_MISS_MATCH | LABEL | Serial number provided does not match EEPROM value. Invalid serial number scanned in |
| FW_MISMATCH | FW_MISS_MATCH | LABEL | The BUT programmed FW Rev does not match FW File. Re-Commission the M1-3200 |
| Lever Sensor | #NAME? | ICT | The Lock Lever is not locked |
| ERR_UNDER | 0000ERR_UNDER | Label | Undefined error, i.e. no error code available in the DB for this failure. Contact LenelS2 |